Lab 10: Sampling & Quantization

Objective

The 1st half of this Lab experiment is basically about how to sample a continuous time signal into discrete without Aliasing Effect. In the 2nd half, the concept of quantization will be discussed. In addition to this you will the basics of analog-to-digital and digital-to-analog conversions.

Lab Tasks

1. Write a generalized program to sample a Band Limited continuous time signal y=cos(2*pi*f1*t) + cos(2*pi*f2*t) that is limited to fm under the following conditions. Also comment on the results in terms of aliasing effect both in time and frequency domain. Show a Time/Frequency comparison of each part with the original signal.

i. fs=fm

ii. fs>2fm (Oversampling)

iii. fs<fm (Undersampling)

CODE:

clear all;

close all;

clc;

% Signal parameters

f1 = 2; % Frequency of the first component (Hz)

f2 = 3; % Frequency of the second component (Hz)

fm = 5; % Maximum frequency of the band-limited signal

% Continuous-time signal (original)

fs_ct = 1000; % Very high sampling frequency to approximate continuous signal

t_ct = 0:1/fs_ct:1; % Time vector for the continuous signal

y_ct = cos(2*pi*f1*t_ct) + cos(2*pi*f2*t_ct);

% Plot original signal

figure;

subplot(4, 1, 1);

plot(t_ct, y_ct, 'LineWidth', 1.5);

title('Original Signal (Continuous Time)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Critical Sampling: fs = fm

fs_crit = fm; % Critical sampling frequency

t_crit = 0:1/fs_crit:1; % Time vector for critical sampling

y_crit = cos(2*pi*f1*t_crit) + cos(2*pi*f2*t_crit);

subplot(4, 1, 2);

stem(t_crit, y_crit, 'filled', 'MarkerSize', 4);

hold on;

plot(t_ct, y_ct, 'r--'); % Overlay the original signal

title('Critically Sampled Signal (fs = fm)');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Sampled', 'Original');

grid on;

% Oversampling: fs > 2fm

fs_over = 20; % Oversampling frequency

t_over = 0:1/fs_over:1; % Time vector for oversampling

y_over = cos(2*pi*f1*t_over) + cos(2*pi*f2*t_over);

subplot(4, 1, 3);

stem(t_over, y_over, 'filled', 'MarkerSize', 4);

hold on;

plot(t_ct, y_ct, 'r--'); % Overlay the original signal

title('Oversampled Signal (fs > 2fm)');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Sampled', 'Original');

grid on;

% Undersampling: fs < fm

fs_under = 2; % Undersampling frequency

t_under = 0:1/fs_under:1; % Time vector for undersampling

y_under = cos(2*pi*f1*t_under) + cos(2*pi*f2*t_under);

subplot(4, 1, 4);

stem(t_under, y_under, 'filled', 'MarkerSize', 4);

hold on;

plot(t_ct, y_ct, 'r--'); % Overlay the original signal

title('Undersampled Signal (fs < fm)');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Sampled', 'Original');

grid on;

RESULTS

2. Consider an analog signal x(t) consisting of three sinusoids of frequencies of 1 Hz, 4 Hz, and 6 Hz:

x(t)= cos(2πt) + cos(8πt) + cos(12πt)

where t is in seconds. Show that if this signal is sampled at a rate of fs = 5 Hz, it will be aliased with the following signal, in the sense that their sample values will be the same as

xa(t)= 3cos(2πt)

On the same graph, plot the two signals x(t) and xa(t) versus t in the range 0 ≤ t ≤ 2 sec. To this plot, add the time samples x(tn) and verify that x(t) and xa(t) intersect precisely at these samples.

These samples can be evaluated and plotted as follows:

e,g. fs = 5000; T = 1/fs; tn = 0:T:2; plot(tn, xn);

Any observation regarding the comparison of both in frequency domain?

CODE:

clear all;

close all;

clc;

% Sampling frequency and signal parameters

fs = 12; % Sampling frequency (must be greater than 2*f3 to avoid aliasing)

f1 = 1;  % Frequency of the first signal (Hz)

f2 = 4;  % Frequency of the second signal (Hz)

f3 = 6;  % Frequency of the third signal (Hz)

% Time vector for sampling

t = 0:1/fs:2;

% Original signals

y = cos(2*pi*f1*t) + cos(2*pi*f2*t) + cos(2*pi*f3*t); % Combined signal

y1 = 3*cos(2*pi*f1*t); % Additional signal

% Combined signal

xn = y + y1;

% Plot the signals

figure;

subplot(3, 1, 1);

plot(t, y, 'LineWidth', 1.5);

title('Original Signal (y)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3, 1, 2);

plot(t, y1, 'LineWidth', 1.5);

title('Additional Signal (y1)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3, 1, 3);

plot(t, xn, 'LineWidth', 1.5);

title('Combined Signal (y + y1)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

RESULT:

3. Repeat above with fs = 10 Hz. In this case, determine the signal xa(t) with which x(t) is aliased. Plot both x(t) and xa(t) on the same graph over the same range 0 ≤ t ≤ 2 msec. Verify again that the two signals intersect at the sampling instants. Your results should look like following:

Also show how both the signals are aliased in frequency domain.

CODE:

% Signal parameters

f1 = 1; f2 = 4; f3 = 6; % Frequencies of the sinusoids (Hz)

% Time vector for high-resolution plot (continuous signal)

t = 0:0.0001:2; % High-resolution time vector (0.1 ms step)

% Original signal x(t)

x_t = cos(2*pi*f1*t) + cos(2*pi*f2*t) + cos(2*pi*f3*t);

%% Case 1: Sampling at fs = 5 Hz

fs1 = 5; % Sampling frequency

T1 = 1/fs1; % Sampling period

tn1 = 0:T1:2; % Discrete sampling instants

% Discrete-time sampled signal values

x_n1 = cos(2*pi*f1*tn1) + cos(2*pi*f2*tn1) + cos(2*pi*f3*tn1);

% Aliased signal xa(t) for fs = 5 Hz

% All frequencies alias back to 1 Hz (f_a = |f - n*fs|)

xa_t1 = 3*cos(2*pi*f1*t);

% Plot for fs = 5 Hz

figure;

subplot(1, 2, 1);

plot(t*1000, x_t, 'b-', 'LineWidth', 1.5); hold on; % x(t)

plot(t*1000, xa_t1, 'r--', 'LineWidth', 1.5); % xa(t)

stem(tn1*1000, x_n1, 'k', 'filled'); % Sampled points

xlabel('t (msec)');

ylabel('x(t), x_a(t)');

title('Sampling at f_s = 5 Hz');

legend('x(t)', 'x_a(t)', 'Samples');

grid on;

%% Case 2: Sampling at fs = 10 Hz

fs2 = 10; % Sampling frequency

T2 = 1/fs2; % Sampling period

tn2 = 0:T2:2; % Discrete sampling instants

% Discrete-time sampled signal values

x_n2 = cos(2*pi*f1*tn2) + cos(2*pi*f2*tn2) + cos(2*pi*f3*tn2);

% Aliased signal xa(t) for fs = 10 Hz

% f3 = 6 Hz aliases to f3_alias = 10 - 6 = 4 Hz

xa_t2 = cos(2*pi*f1*t) + 2*cos(2*pi*f2*t); % Add aliasing contribution

% Plot for fs = 10 Hz

subplot(1, 2, 2);

plot(t*1000, x_t, 'b-', 'LineWidth', 1.5); hold on; % x(t)

plot(t*1000, xa_t2, 'r--', 'LineWidth', 1.5); % xa(t)

stem(tn2*1000, x_n2, 'k', 'filled'); % Sampled points

xlabel('t (msec)');

ylabel('x(t), x_a(t)');

title('Sampling at f_s = 10 Hz');

legend('x(t)', 'x_a(t)', 'Samples');

grid on;

RESULTS

4. Consider a 100 Hz sine wave x(n) is sampled with fS = 4 kHz, N = 64 number of samples and w = 3 bit (word-length).

• What is the number of quantization levels?

• What is the quantization step size Q if the signal is normalized to -1 < x(n) < 1?

• Show graphically how quantization is performed. What is the maximum error for this 3-bit quantizer? What will the trend of quantization error if number of bits are increased to 5?

ANSWER

The number of quantization levels (L) for an w-bit quantizer is given by:

L = 2w

For w = 3

L = 23 = 8

If the signal

x(n) is normalized to the range −1 ≤ x(n) ≤ 1 , the quantization step size (𝑄Q) is calculated as:

Q = Range/Number of levels = 1−(−1)/𝐿 = 2/8  =0.25

So, the quantization step size Q=0.25.

CODE:

% Parameters

f = 100; % Frequency of sine wave (Hz)

fs = 4000; % Sampling frequency (Hz)

N = 64; % Number of samples

w = 3; % Word length (bits)

L = 2^w; % Number of quantization levels

Q = 2 / L; % Quantization step size

% Time vector and sine wave

n = 0:N-1;

x_n = sin(2*pi*f*n/fs); % Sine wave sampled at fs

% Quantization

x_q = round(x_n / Q) * Q; % Quantized signal

% Plot

figure;

subplot(2, 1, 1);

plot(n, x_n, 'b-', 'LineWidth', 1.5); hold on;

stem(n, x_q, 'r', 'filled', 'LineWidth', 1); % Quantized signal

xlabel('Sample Index');

ylabel('Amplitude');

title('Quantization of Sine Wave');

legend('Original Signal', 'Quantized Signal');

grid on;

% Quantization error

error = x_n - x_q;

subplot(2, 1, 2);

stem(n, error, 'k', 'filled');

xlabel('Sample Index');

ylabel('Error');

title('Quantization Error');

grid on;

RESULTS:

The maximum quantization error (Emax​) for a uniform quantizer is:

Emax​ = Q/2 ​= 0.25/2 ​= 0.125

If the word length (www) increases:

- The number of quantization levels (L) increases, reducing the step size (Q).

- The quantization error decreases as Q gets smaller.

For 𝑤=5:

𝐿 = 25 = 32

Q = 2/32 = 0.0625

Emax ​= Q/2 ​= 0.03125

This indicates that increasing the word length improves the resolution of the quantizer and reduces the quantization error.