Lab 11: Audio/Speech Processing in MATLAB

Objectives:

• Familiarizing students with processing of speech signals in MATLAB.

Lab Tasks:

1. Use MATLAB to read an audio signal, play it, obtain its sampled data, and plot this signal.

CODE:
% Read the audio signal

fs=4410;

[data, fs] = audioread('D:\5th SEM\SNS\lab11 sol\Yokoso Watashi no Soul society.mp3

'); % Replace 'audiofile.wav' with your file name

% Play the audio

sound(data, fs);

% Plot the signal

t = (0:length(data)-1) / fs; % Time vector

plot(t, data);

xlabel('Time (s)');

ylabel('Amplitude');

title('Audio Signal');

grid on;

RESULT:

2. Use MATLAB to record a voice signal, play it, obtain its sampled data, and plot this signal.

CODE:
% Record the voice signal

fs = 44100; % Sampling frequency

duration = 5; % Duration in seconds

recObj = audiorecorder(fs, 16, 1); % Create audio recorder

disp('Recording... Speak now.');

recordblocking(recObj, duration); % Record for the specified duration

disp('Recording complete.');

% Play the recorded audio

sound(getaudiodata(recObj), fs);

% Plot the signal

data = getaudiodata(recObj); % Get audio data

t = (0:length(data)-1) / fs; % Time vector

plot(t, data);

xlabel('Time (s)');

ylabel('Amplitude');

title('Recorded Voice Signal');

RESULT:

3. Download two test audio files and use MATLAB to obtain the sampled data for these two audio signals. Subtract signal 2 from signal 1. Use subplot command to plot the original signals and the resultant signal.

CODE:

% Step 1: Read the two audio signals

[data1, fs1] = audioread('D:\5th SEM\SNS\lab11 sol\Hisashiburi dana Mugiwara.mp3'); % Replace 'audio1.wav' with your file name

[data2, fs2] = audioread('D:\5th SEM\SNS\lab11 sol\daijoubu desu datte kimi yowai mo   gojo Satoru.mp3'); % Replace 'audio2.wav' with your file name

fs1 =44100;

fs2 =44100;

% Check if sampling frequencies match

if fs1 ~= fs2

error('Sampling frequencies of the two audio files do not match.');

end

% Ensure both signals have the same length

minLength = min(length(data1), length(data2));

data1 = data1(1:minLength, :); % Truncate to the shortest length

data2 = data2(1:minLength, :); % Truncate to the shortest length

% Step 2: Subtract signal 2 from signal 1

resultantSignal = data1 - data2;

% Step 3: Plot the signals

t = (0:minLength-1) / fs1; % Time vector

figure;

% Plot Signal 1

subplot(3, 1, 1);

plot(t, data1,'g');

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal 1');

grid on;

% Plot Signal 2

subplot(3, 1, 2);

plot(t, data2);

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal 2');

grid on;

% Plot Resultant Signal

subplot(3, 1, 3);

plot(t, resultantSignal,'y');

xlabel('Time (s)');

ylabel('Amplitude');

title('Resultant Signal (Signal 1 - Signal 2)');

grid on;

RESULT:

4. Download two test audio files and use MATLAB to concatenate the first file with the second file.

Use subplot command to plot the original signals and the resultant signal.

(Hint: The order of concatenation should be such that the resultant signal should have signal 1 first

and signal 2 right after signal 1.)

CODE:

% Step 1: Read the two audio signals

fs1 = 44100;

fs2 = 44100;

[data1, fs1] = audioread('D:\5th SEM\SNS\lab11 sol\Hisashiburi dana Mugiwara.mp3'); % Replace with your first audio file

[data2, fs2] = audioread('D:\5th SEM\SNS\lab11 sol\Ore Wa Kaizoku-ou Ni Naru Otoko.mp3'); % Replace with your second audio file

% Step 2: Check if sampling frequencies match

if fs1 ~= fs2

error('Sampling frequencies of the two audio files do not match.');

end

% Step 3: Concatenate the signals

resultantSignal = [data1; data2];

% Step 4: Create time vectors

t1 = (0:length(data1)-1) / fs1; % Time vector for Signal 1

t2 = (0:length(data2)-1) / fs2; % Time vector for Signal 2

tResult = (0:length(resultantSignal)-1) / fs1; % Time vector for the resultant signal

% Step 5: Plot the signals

figure;

% Plot Signal 1

subplot(3, 1, 1);

plot(t1, data1);

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal 1');

grid on;

% Plot Signal 2

subplot(3, 1, 2);

plot(t2, data2);

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal 2');

grid on;

% Plot Resultant Signal

subplot(3, 1, 3);

plot(tResult, resultantSignal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Concatenated Signal (Signal 1 followed by Signal 2)');

grid on;

sound(resultantSignal, fs1);

RESULTS