Lab 7: Fourier Series

Objectives:

• Familiarizing students with periodic signals.

• Calculate the Fourier series of continuous and discrete time signals and plot the Fourier series coefficients.

Lab Tasks:

1. Consider the given function 𝑥(𝑡) where:

𝑥(𝑡) = 𝑡2 𝑓𝑜𝑟 − 1 ≤ 𝑡 ≤ 1

Calculate the Fourier series coefficients (𝑎𝑘 , 𝑏𝑘 and 𝑎0) using MATLAB. Use these coefficients

to approximate 𝑥(𝑡).

ANSWER:

syms t k;

% Define the function x(t) = t^2

x = t^2;

% Define the period T = 1

T = 1;

% Compute the a0 coefficient (mean value)

a0 = (1/T) * int(x, t, -1, 1);

% Compute the ak coefficient for k = 1, 2, 3, ...

ak = (2/T) * int(x * cos(k * pi * t / T), t, -1, 1);

% Compute the bk coefficient for k = 1, 2, 3, ...

bk = (2/T) * int(x * sin(k * pi * t / T), t, -1, 1);

% Display the Fourier coefficients a0, ak, and bk

disp(['a0 = ', char(a0)]);

disp(['ak = ', char(ak)]);

disp(['bk = ', char(bk)]);

% Fourier series approximation using the first 5 terms

x_approx = a0 / 2; % Start with a0/2

n_terms = 5; % Number of terms

for n = 1:n_terms

% Add terms for Fourier series

x_approx = x_approx + double(subs(ak, k, n)) * cos(n * pi * t) + double(subs(bk, k, n)) * sin(n * pi * t);

end

% Plot the original function and Fourier series approximation

fplot(x, [-1, 1], 'LineWidth', 2); hold on;

fplot(x_approx, [-1, 1], '--', 'LineWidth', 2);

legend('Original x(t)', 'Fourier Series Approximation');

title('Original Function and Fourier Series Approximation');

xlabel('t');

ylabel('x(t)');

grid on;

RESULT:

2. Approximate the square wave using Fourier Series Coefficients and plot the approximations by using 1, 3, 9, 21, 49, 71, 121, 211 & 516 coefficients.

ANSWER
% Define the time variable

t = linspace(-1, 1, 1000); % Time interval from -1 to 1

% Define the square wave function (periodic square wave)

square_wave = @(t) sign(sin(pi * t)); % Square wave with period 2

% Fourier Series approximation

% Initialize the figure

figure;

hold on;

% Plot the original square wave

plot(t, square_wave(t), 'k', 'LineWidth', 2, 'DisplayName', 'Original Square Wave');

% List of number of terms to use in the approximation

terms = [1, 3, 9, 21, 49, 71, 121, 211, 516];

% Loop through each number of terms

for n_terms = terms

% Initialize the Fourier series approximation

approximation = zeros(size(t));

% Sum the Fourier series up to n_terms

for k = 1:n_terms

if mod(k, 2) == 1  % Only odd harmonics (1, 3, 5, ...)

approximation = approximation + (4 / (pi * k)) * sin(pi * k * t);

end

end

% Plot the approximation

plot(t, approximation, 'LineWidth', 1.5, 'DisplayName', ['N = ', num2str(n_terms)]);

end

% Add labels and legend

xlabel('t');

ylabel('Amplitude');

title('Square Wave Approximation using Fourier Series');

legend('show');

grid on;

hold off;

RESULT