Lab 8: Fourier Transform

OBJECTIVES

- Fourier Transform (FT)

- Discrete Fourier transform (DFT)

- Discrete Time Fourier transform (DTFT)

EXERCISES

1) Find and plot the Fourier Transform of the following signals:

ANSWER

syms t w

% Define the signals

g1 = exp(-t) * heaviside(t);  % g(t) = e^(-t) for t > 0

g2 = exp(-abs(t));            % g(t) = e^(-|t|)

% Fourier Transform

G1_f = fourier(g1, t, w);

G2_f = fourier(g2, t, w);

% Display results

disp('Fourier Transform of g(t) = e^(-t) for t > 0');

disp(G1_f);

disp('Fourier Transform of g(t) = e^(-|t|)');

disp(G2_f);

% Plot

f_vals = -10:0.1:10;

G1_vals = subs(G1_f, w, f_vals);

G2_vals = subs(G2_f, w, f_vals);

figure;

subplot(2, 1, 1);

plot(f_vals, abs(G1_vals));

title('Fourier Transform of g(t) = e^{-t} for t > 0');

xlabel('Frequency (f)');

ylabel('|G(f)|');

subplot(2, 1, 2);

plot(f_vals, abs(G2_vals));

title('Fourier Transform of g(t) = e^{-|t|}');

xlabel('Frequency (f)');

ylabel('|G(f)|');

RESULT:

2) Find and plot the Discrete Fourier Transform of the following signals:

ANSWER

% Define signals

x = [1, 1, 0, 1];

y = [5, 4, 3, 2, 1, 0];

% DFT using FFT

X = fft(x);

Y = fft(y);

% Plot

figure;

subplot(2, 1, 1);

stem(0:length(X)-1, abs(X));

title('DFT of x[n] = [1, 1, 0, 1]');

xlabel('Frequency Index');

ylabel('|X[k]|');

subplot(2, 1, 2);

stem(0:length(Y)-1, abs(Y));

title('DFT of y[n] = [5, 4, 3, 2, 1, 0]');

xlabel('Frequency Index');

ylabel('|Y[k]|');

RESULTS

3) Find and plot the Discrete Time Fourier Transform of the following signal:

ANSWER
syms n f

x_n = (0.5)^n * heaviside(n);

% DTFT

X_f = symsum(x_n * exp(-1j * 2 * pi * f * n), n, 0, inf);

% Display and plot

f_vals = -0.5:0.01:0.5;

X_f_vals = double(subs(X_f, f, f_vals));

figure;

plot(f_vals, abs(X_f_vals));

title('DTFT of x[n] = (0.5)^n u[n]');

xlabel('Frequency (f)');

ylabel('|X(f)|');

RESULT

4) Write a program to calculate the inverse Fourier transform of a signal and apply it on part 1 to recover the original signals given.

ANSWER

syms t w

% Define the signals

g1 = exp(-t) * heaviside(t);  % g(t) = e^(-t) for t > 0

g2 = exp(-abs(t));            % g(t) = e^(-|t|)

g3 = heaviside(t);

% Fourier Transform

G1_f = fourier(g1, t, w);

G2_f = fourier(g2, t, w);

G3 = fourier(g3,t,w);

% Inverse Fourier Transform

f1=ifourier(G1_f,w,t);

f2=ifourier(G2_f,w,t);

% Display results

disp('Inverse Fourier Transform of G1_f');

disp(f1);

disp('Inverse Fourier Transform of G2_f');

disp(f2);

disp(G3);

RESULT:

5) Write a program to calculate the inverse discrete Fourier transform of a signal and apply it on part 2 to recover the original signal given.

ANSWER

% Inverse DFT for x[n] and y[n]

x_recovered = ifft(X);

y_recovered = ifft(Y);

disp('Recovered x[n] from IDFT:');

disp(x_recovered);

disp('Recovered y[n] from IDFT:');

disp(y_recovered);

RESULT:

6) Write a program to calculate the inverse discrete time Fourier transform of a signal and apply it on part 3 to recover the original signals given.

ANSWER

N = 256;  % Large number of points

k = 0:N-1;

f_vals = k/N - 0.5;

% Compute sampled X(f) for DTFT approximation

X_f_vals = double(subs(X_f, f, f_vals));

% IDFT to recover x[n]

x_recovered = ifft(X_f_vals) * N;

disp('Recovered x[n] from IDTFT approximation:');

disp(real(x_recovered(1:10)));  % Display first 10 values

RESULT: