Lab 9: Z-Transform & Inverse Z-Transform

Objective

In this Lab our aim is to become familiar with Z-transform and its properties, Inverse of Z-transform & solving difference equations using Z-transform. The Lab is divided into two sections. Section A deals with Z-Transform while inverse Z-Transform is discussed in Section B.

Getting help in MATLAB

There are several sources from which help can be obtained. Three of the most commonly used are represented below. It is strongly recommended that you explore these thoroughly before proceeding with the other exercises, since they will be useful for all remaining laboratory exercises for DSP.

MATLAB help window – type “helpwin” at the command prompt. This will launch a new window where all MATLAB core functions are categorized. To find a function, select the appropriate category and select the operator you need to learn more about its syntax and operation.

MATLAB help desk – to launch, type “helpdesk” at the command prompt. The help desk will give you web pages with more detailed help documents and examples of how functions can be used.

MATLAB function help - by typing in “help <function_name>” you can receive help on a particular function. This is useful if you know the name of the function but are not sure about the syntax.

MATLAB demos – type in “demo” at the command prompt. This takes you to a window where simple demos show different capabilities of the software. This is a good starting point for new users. Throughout out this lab, use the MATLAB beginner’s manual and the MATLAB help pages as reference.

Section A Z-Transform

The z-transform is useful for the manipulation of discrete data sequences and has acquired a new significance in the formulation and analysis of discrete-time systems. It is used extensively today in the areas of applied mathematics, digital signal processing, control theory, population science, and economics. These discrete models are solved with difference equations in a manner that is analogous to solving continuous models with differential equations. The role played by the z-transform in the solution of difference equations corresponds to that played by the Laplace transforms in the solution of differential equations.

The z-transform of a sequence x(n) is given by:

Where z is a complex variable. The set of z values for which X (z) exists is called the region of convergence (ROC). To recover [𝑛] from (𝑧), we use the inverse Z-transform.

The complex variable z is called the complex frequency given by 𝑧=|𝑧|ejw where |z| is the attenuation and 𝜔 is the real frequency. The function |z| = 1 (or z = ejw) is a circle of unit radius in the z-plane and is called the unit circle. If the ROC contains the unit circle, then we can evaluate X(z) on the unit circle.

The output of a system 𝑦[𝑛] in Z-domain is given by 𝑌(𝑧). The system function ℎ[𝑛] is given by 𝐻(𝑧), and the input 𝑥[𝑛] is given by 𝑋(𝑧). We can define the output as 𝑌(𝑧)=𝐻(𝑧)𝑋(𝑧)

and the system function can be computed by 𝐻(𝑧)=𝑌(𝑧)𝑋(𝑧)

The system function can be written as 𝐻(𝑧)=𝑏𝑜(𝑧−𝑧1)(𝑧−𝑧2)…(𝑧−𝑧𝑤−1)(𝑧−𝑝1)(𝑧−𝑝2)…(𝑧−𝑝𝑤)

where 𝑍𝑖 represent the zeros and Pm represent the poles of the system function. To find the zeros and poles of a system, we can use the MATLAB’s 𝒓𝒐𝒐𝒕𝒔() function. For a causal system, the system is stable if and only if the transfer function has all its poles inside the unit circle. i.e. |𝑝𝑚|<1, 𝑚=1,2,…𝑊 The poles can be viewed via the MATLAB’s 𝒛𝒑𝒍𝒂𝒏𝒆() command. The screen shot of location of poles and zeros for a moving-average filter is shown in Figure below. The unfilled circles represent zeros and the crosses represent the poles of the system.

The frequency response of a system can be determined from its system function by replacing Z with 𝑒𝑗𝜔 and then converting 𝐻(𝑧) to polar form. 𝐻(𝑧=𝑒𝑗𝜔)=𝐻(𝑒𝑗𝜔)=|𝐻(𝑒𝑗𝜔)|∠(𝐻(𝑒𝑗𝜔))

The frequency response is then subdivided into magnitude and phase response. The frequency response of a digital system can be computed by the MATLAB’s function freqz().

The MATLAB’s FVTool can also be used to compute the response of a digital filter. Not only the magnitude and phase responses are provided to the user, but quantities like group delay and phase delay can also be dealt with easily using this GUI. The Figure below shows the screen shot of the FVTool.

We see that the quantities that FVTool can provide us are:

• The Magnitude Response

• The Phase Response

• Superimposes of magnitude and phase response

• Group delay of the filter

• Phase delay of the system

• Impulse response of the system

• Step response of the current filter

• Pole-zero plot of the filter

• The filter coefficients of the current filter

Thus, we can see that the FVTool is a powerful tool for analyzing the digital filters.

Questions

1. Under what conditions a system is stable & causal according to PZ analysis.

Answer:

 Stability: All system poles must lie inside the unit circle in the z-plane.

 Causality: The system's Region of Convergence (ROC) must include the unit circle and extend outward (right-sided ROC).

2. Under what conditions the inverse system of a stable system is also stable?

Answer: The inverse system of a stable system is stable only if the poles of the inverse system (which correspond to the zeros of the original system) also lie inside the unit circle.

3. Why we move towards Z-Transform although we already are having different flavors of Fourier Transforms?

Answer: Z-Transform is used because it provides a unified framework for analyzing discrete-time systems, capturing both transient and steady-state behavior, unlike Fourier transforms, which are limited to frequency-domain analysis and assume infinite time duration.

4. Differentiate between Laplace Transform & Z-Transform.

Answer:

 Laplace Transform: Continuous-time systems, s-plane, applicable to continuous signals.

 Z-Transform: Discrete-time systems, z-plane, applicable to discrete signals.

5. Write an equation that describes the relationship between Laplace, Fourier and Z- Transform.

Answer:

 Laplace Transform: s=σ+jωs  (continuous).

 Z-Transform: z=esT, where T is the sampling period.

 Fourier Transform: Derived from both Laplace (s=jω) and Z-Transform (z=ejω).